MIDLANDS LUBRICANTS LTD.

Technical Data Sheet

Cutting Oil 22S

Product Description

Midlands Lubricants Cutting Oil 22S is a medium viscosity cutting oil containing lubricity additives. It is suited for general machining operations on ferrous metals up to medium tensile steels and non-ferrous metals. The additive system ensures improved surface finish and tool life whilst remaining non-staining to Copper and it’s alloys. Due to the base oil and additive selections the product is pale in colour, low odour and low viscosity in use.

Features/Benefits:

Low Mist

Non-staining

Multi Metal Applications

Pale Colour

Low Odour

Typical Physical Properties:

Density, g/cm3 @ 15.5°C (60°F) 0.862

Flash Point, COC                       210°C

Pour Point, COC                        -12°C

Viscosity

Kinematic, 40°C                         22 cSt

Kinematic, 100°C                       4.2 cSt

Viscosity Index                           96

These characteristics are typical of current production material, whilst further production will

meet these specifications, some variation may occur.