MIDLANDS LUBRICANTS LTD.

Technical Data Sheet

20w/50 Mineral Engine Oil

Product Description

20W/50 is a traditional mineral oil based engine oil suitable for use in gasoline fuelled passenger cars and light duty diesel vehicles. It is formulated from a blend of top quality solvent refined mineral base oils with established detergent / dispersant additive technology and incorporating viscosity index improver polymers to deliver the 20W/50 multi-grade viscosity profile. The 20W/50 viscosity grade classification is designed for severe service conditions and ensures optimum lubrication protection and engine performance in warmer climate conditions.

FEATURES / BENEFITS

High Temperature Performance Protects against Ring Sticking at High Temperatures Maintains Oil Performance Minimises Oil Consumption High Standard of Engine Cleanliness Excellent Sludge Control Protection designed for Stop-Go City Motoring Long Lasting Dispersant, Anti-Wear and Corrosion Protection

PERFORMANCE / SPECIFICATIONS

- API: SL/CF

- ACEA: A3/B3

- MB: 229

TYPICAL PROPERTIES

- Total Base Number (TBN) 9 mgKOH.g-1

- Density, g/cm3 @ 15°C 0.885

- Flash Point, COC 220°C

- Pour Point -30°C

- Viscosity

- Kinematic, @ 40°C 153 cSt

- Kinematic, @ 100°C 18.2 cSt

- Viscosity Index 133